MENDOTA DUAL STEP

7100 Spectrum Lane | Missoula MT 59808 | 800.791.8056 | www.spectrumproducts.com

Mendota Dual Step Assembly:

Tools Required;

Drill with Square Drive Bit

1/2" Open End Wrench

1/2" Socket and Ratchet Mallet

Hardware Pack Includes;

(22) 2 1/2" Square Drive Screws

(18) 1 1/2" Square Drive Screws

(26) 5/8" Carriage Bolts

(26) 5/8" Hex Nuts

Note:

- All carriage bolts to be left loose until stated otherwise.

Set up chair on a flat surface, this will make the installation of the hardware much simpler.

- Set the heads of the carraige bolts into the HDPE lumber with the mallet beforetightening; DO NOT overtighten the hex nuts.

Step 1:

Using the provided pre-drilled holes attach the Front Platform Support to the Front End Brace with; (4) 2 1/2" Square Drive Screws.

Step 2:

Attach the base with;

(2) Carriage Bolts

(2) Hex Nuts

Note - do not tighten hardware leave loose until stated otherwise

Step 3:

Place Lower Back as shown. Attach with;

(2) Carriage Bolts

(2) Hex Nuts

Step 4:

Attach the Platform to Front Platform

Support with;

(2) Carriage Bolts

(2) Hex Nuts

Swing Foot Rest up to meet Lower

Back.  Attach with;

(4) Carriage Bolts

(4) Hex Nuts

Add screw to Platform and Front Platform Support using pre drilled holes.

(2) 2 1/2" Square Drive Screws

Step 5:

Place Arm Supports as shown. Attachment at Base uses;

(2) Carriage Bolts

(2) Hex Nuts

Attachment to Platform uses;

(4) Carriage Bolts

(4) Hex Nuts

- Base

Step 6:

Attach Seat to Arm Support using;

- Carriage Bolts

(2) Hex Nuts

Swing Foot Rest up to meet Lower

Back.  Attach with;

(4) Carriage Bolts

(4) Hex Nuts

- Lower Back

Use these pre-drilled holes.  On top two holes use front hole

Step 7:

Attach Upper Back to Seat as Shown Below;

- 1 1/2" Square Drive Screws

Attachment to Lower Back using;

(4) 2 1/2" Square Drive Screws

(1) Lower Back

Flush top and side Step 8:

Attach Arm Stiffener to Lower Back. Use provided pre drilled holes;

(4) 1 1/2" Square Drive Screws

Note:  Insure Arm Stiffener is flush to Lower Back top and side.

Flush front of Arm Stiffener to Arm Support than attach with;

(4) 1 1/2" Square Drive Screws

(1) Lower Back Step 7:

Tighten all hardware.  DO NOT overtighten hex nuts.

Step 9:

Place Step Supports as shown.  Attach to Base using;

(4) 2 1/2" Square Drive Screws

Attach Steps to Step Supports and

Front End Brace with;

(4) Carriage Bolts

(4) Hex Nuts

Attach Steps to Front Platform Support and Arm Support with provided pre drilled holes using;

(8) 2 1/2" Square Drive Screws

  - Base

  - Step Support

Flush to rear of Lower Back

(2) Arm Stiffener Step 10:

Attach Arm Rest to Arm Stiffener.

Use provided pre drilled holes and;

(8) 1 1/2" Square Drive Screws

Note:  Insure Arm Rest parallel to Arm Stiffener and Flush to the rear of the Lower Back.

Step 11:

Attach Wheels using;

(2) Bolt CB 5/16"-18x3 1/2"

(4) Flat Washer 5/16"

(2) 5/16" Nylock Nut