Mendota 60” & 66"

Portable Lifeguard Chair

7100 Spectrum Lane | Missoula MT 59808 | 800.791.8056 | www.spectrumproducts.com

Mendota 60"-66” Assembly:

Tools Required:

Drill with Quad Head Bit

1/2” Open End Wrench

1/2” Socket and Ratchet

Mallet

No bolt necessary on this model as shown here

# DETAIL A

- Assemble main frame together as shown on one side in Detail A, making sure to leave hardware loose, using:

5/16” x 3 ½” Carriage bolts

5/16” Lock Washers

5/16” Hex Nuts

*Note:  Set up the chair on a flat surface, this will make installation of hardware much simpler.

*Note:  Set the heads of the carriage bolts into the HDPE lumber with the mallet before tightening; do not over tighten the hex nuts.

Armrest Stiffener

Armrest Stiffener

As shown in Detail B, screw together the armrest stiffener (both sides) making sure to screw into the existing holes, using:

- - 1 1/2" Quad Head Screws

*Note:  It is important not to over tighten the stainless-steel screws, this leaves a depression in the surface where water and contaminates can build up.

- As shown in Detail C, attach seat back to seat bottom (please use existing holes) using:

2 – 1 1/2" Quad Head Screws

2 1/2" Quad

1

½”

Quad Head

Screws

Head Screws

# DETAIL D                                           DETAIL C

- Attach the seat back as shown in Detail D making sure to screw into existing holes, using:

DETAIL E

4 – 2 ½” Quad Head Screws

- Then attach armrest as shown in Detail E using:

4 –1 ½” Quad Head Screws

- For 60" Mendota Slide Base Support in as shown.

- Attach wheels as shown using;

2 -  BOLT CB 5/16"-18x3 1/2" TYPE 316

4 -  FLAT WASHER 5/16"

2 -  5/16" NYLOCK NUT