# RECALL ROPE KIT – 100’

# ASSEMBLY INSTRUCTIONS

# P/N 23650-00

Use the diagram to help in assembling the kit.

- Verify you have received all the required components.  Your kit should contain the following:

  - Item 57050-00 3/8-inch Yellow Braided Rope 			100 feet

  - Item 57260-00 Red 3x5 Poly Floats 				3 ea

  - Item 58180-00 5/8-inch x 3/4-inch Swivel Lever Rope Hook 	1 ea

  - Item 23652-00 1/2-inch Trigger Snap 				1 ea

  - Item 23651-00 3/8-inch Fibrous Rope Clamps			3 ea

- Using a Phillips Head Screwdriver, remove the two screws in one rope clamp and separate the two halves.

- Loop one end of the 3/8-inch rope through the swivel lever hook as shown in the diagram.

- Press the long end of the 3/8-inch rope into one half of the rope clamp as shown.  The long end of the rope goes through both ends of the rope clamp.  Allow about 4 to 6-inches of rope between the swivel lever hook and the rope clamp.

- Press the short end of the 3/8-inch rope into the remaining slot in the rope clamp half.

- Re-assemble the clamp halves together and secure with the two screws removed in step 2.  Pull on both ends of the rope to make sure the connection is secure.

- Cut the 3/8-inch rope to length as needed leaving approximately 8 to 12-inches to make the end loop.

- Thread the 3/8-inch rope through the three poly floats as shown in the diagram.

- Repeat steps 2 through 6 to close the end of the rope.