Top 10 Ways to Keep Your Pool Lift Operational

1. Follow the Manufacturer’s Maintenance Schedule

Every lift model has specific maintenance guidelines in its manual. Stick to the recommended inspection and service intervals this is the foundation of long-term reliability.

2. Charge the Battery Properly (and Regularly)

Most lifts, like those from Spectrum Aquatics, use rechargeable batteries.

Best practices:

- Charge nightly or whenever not in use

- Avoid letting batteries fully drain

- Keep spare batteries charged and rotated

- Store batteries in a dry, temperature-controlled area

A weak battery is the #1 cause of lift failure.

3. Rinse After Exposure to Pool Chemicals

Chlorine and saltwater are harsh on metals and electronics.

- Rinse the lift with fresh water regularly

- Pay special attention to moving joints and hardware

- Wipe dry when possible to reduce corrosion

4. Perform a Weekly Visual Inspection

Catch small issues before they become expensive repairs.

Look for:

- Loose bolts or hardware

- Frayed straps

- Cracks in the seat or footrest

- Corrosion on metal components

- Unusual noises during operation

5. Test the Lift Daily

Before opening your facility:

- Run the lift through a full up/down cycle

- Confirm smooth movement

- Check that emergency stop functions properly

- Ensure the handset/controller works

If it hesitates, jerks, or sounds strained — investigate immediately.

6. Keep the Anchor Socket Clean

Debris, sand, and standing water can collect in the deck anchor.

- Flush out debris regularly

- Use an anchor cover when the lift is removed

- Make sure drainage is functioning properly

A clean anchor ensures smooth rotation and proper alignment.

7. Lubricate Moving Parts (When Recommended)

Only use manufacturer-approved lubricants. Over-lubricating can attract debris, while under-lubricating increases wear.

Follow the manual — don’t guess.

8. Protect the Lift During Off-Season

If your facility closes seasonally:

- Remove and store the lift indoors

- Keep it in a dry, climate-controlled area

- Fully charge batteries before storage

- Cover the lift to prevent dust accumulation

This significantly extends the life of the equipment.

9. Train Staff on Proper Operation

Improper use shortens lifespan.

Ensure staff know:

- Safe weight limits

- Proper positioning

- How to operate emergency controls

- How to report issues immediately

A well-trained team prevents avoidable damage.

10. Schedule Annual Professional Service

Even if everything seems fine, have a qualified technician inspect the lift annually.

Professional service helps:

- Maintain ADA compliance

- Validate load capacity

- Replace worn components

- Prevent unexpected breakdowns